🚀 TrustCam-DTN: End-to-End Technical Summary

🔹 What the system does (1-line pitch)

TrustCam-DTN securely captures images at the device, encrypts them before transmission, attaches provenance proofs, and delivers them reliably using Delay-Tolerant Networking — even without internet — while maintaining tamper-evident logs.

🧩 System Flow (Step-by-Step)

1️⃣ Secure Capture at ESP32-CAM

- Camera captures an image into framebuffer

- Before storing or transmitting it, we encrypt it using AES-GCM

- This ensures confidentiality + integrity (via auth-tag)

🧠 Why AES-GCM?

It provides both encryption and authentication without needing a separate HMAC, and it’s fast enough for embedded devices.

2️⃣ Provenance and Authenticity

After encryption, we generate:

- SHA-256 hash of encrypted file

- ECDSA signature over that hash

- Metadata: timestamp, device ID, firmware version

This proves:

| Property | Mechanism |
| --- | --- |
| Who captured it | Signature w/ device private key |
| When it was captured | Timestamp in metadata |
| Whether modified | Hash + signature mismatch if altered |

🧠 Expected viva question:
Q: Why sign after encryption instead of before?
✔ Because signing plaintext leaks info about raw data—signing ciphertext protects structure and maintains privacy.

3️⃣ Local Transparency Log

Each capture event is appended into a hash-chained log stored on device:

(prev_hash + event_hash) → SHA-256 → new entry

It behaves like a lightweight blockchain:

- No consensus

- No mining

- Just immutability

🧠 Viva Q:
Q: Why not use a full blockchain like Ethereum?
✔ Because blockchain is overkill for an IoT device. Hash chaining gives tamper evidence with near-zero resource cost.

4️⃣ Delay-Tolerant Networking (DTN)

Instead of assuming continuous internet connection, we use:

✔ Store-carry-forward model (BPv7 Bundle Protocol)
✔ Messages are queued if network unavailable
✔ Delivered automatically when a route appears
✔ Can integrate satellite (Iridium) later

🧠 Viva Q:
Q: How is this different from normal TCP/IP transmission?
✔ TCP requires continuous end-to-end connectivity.
✔ DTN can deliver data hours or days later, across disconnected nodes.

5️⃣ Proxy-Friendly, Zero-Trust Transport

Because encryption happens at the source:

- DTN nodes

- Servers

- Gateways

…cannot see or alter plaintext.

The server stores encrypted data but cannot read it unless authorized.

🧠 Viva Q:
Q: Why is this better than WhatsApp-style end-to-end encryption?
✔ WhatsApp encrypts messages during transport, but server still controls identity, routing, and metadata.
✔ TrustCam enforces:

- Provenance

- Tamper evidence

- Secure re-sharing (PRE)

- Offline delivery

…none of which WhatsApp guarantees.

6️⃣ Backend Verification + Decryption (Flask)

When data arrives:

| Step | Operation |
| --- | --- |
| 1 | Compute new hash |
| 2 | Verify ECDSA signature |
| 3 | Verify transparency log chain |
| 4 | Decrypt AES-GCM |
| 5 | Store reconstructed image |

If any step fails → backend rejects data.

🧠 Viva Q:
Q: Can the server modify logs or images?
✔ No — modification breaks signature and hash-chain integrity.

7️⃣ Testing and Validation

We tested:

- 🧪 Raw HTTP uploads

- 🧪 Curl-based multipart uploads

- 🧪 Retrieval via /images and /images/<file>

- 🧪 Log verification via /log/verify

Everything worked end-to-end.

🎓 Key Viva Talking Points

| Category | Bullet Answer |
| --- | --- |
| Confidentiality | AES-GCM ensures encryption + authentication |
| Integrity | SHA-256 hash + signature prove no tampering |
| Authenticity | ECDSA keypair binds image to specific device |
| Non-Repudiation | Device cannot deny generating signed image |
| Tamper Detection | Hash-chained logs detect alterations |
| Offline Support | DTN ensures store-and-forward resilience |
| Zero-Trust Architecture | Server does not need to trust devices |

💬 Expected Deep Questions & Strong Answers

🔥 Q: What happens if someone modifies a single byte in the encrypted payload?

AES-GCM authentication tag will fail, signature verification will fail, and log entry will mismatch — system rejects the bundle.

🔥 Q: How do you handle replay attacks?

Metadata includes timestamp + device ID. Backend rejects duplicates or stale entries.

🔥 Q: Why not just use HTTPS?

HTTPS protects transport only while online.
TrustCam protects:

- before transmission

- during storage

- during routing

- without internet

🔥 Q: What’s the biggest bottleneck?

- Cryptographic operations on ESP32 if scaling many images.

- DTN routing latency when no direct route exists.

🏁 Final One-Sentence Conclusion

TrustCam-DTN is a zero-trust secure imaging platform combining encryption-at-source, cryptographic provenance, tamper-evident logging, and delay-tolerant delivery—ensuring images remain verifiable and confidential from capture to final storage, even in disconnected environments.